Here’s a complete, step-by-step guide to wiring your load cell, programming the Arduino, and connecting it to the Python app you’re using.

### 🔌 1. Hardware Setup

A raw load cell cannot connect directly to an Arduino. You must use an HX711 amplifier module.

#### Wiring Diagram (Load Cell → HX711 → Arduino)

| Load Cell Wire | HX711 Pin | Arduino Pin |
| --- | --- | --- |
| Red (E+) | E+ | 5V |
| Black (E-) | E- | GND |
| White (A+) | A+ | D2 |
| Green (A-) | A- | D3 |
| (Optional shield wire) | GND | GND |

💡 Note: Wire colors can vary by manufacturer. Always check your load cell’s datasheet. If the readings jump or show negative, swap the A+ and A- wires.

### 💻 2. Arduino Firmware Setup

#### A. Install the HX711 Library

- Open the Arduino IDE

- Go to Sketch > Include Library > Manage Libraries

- Search for HX711 by bogde

- Click Install

#### B. Upload the Arduino Code

This sketch is specifically formatted to match what your Python app expects (WEIGHT: , TARE_COMPLETE, HX711_NOT_READY, and T for tare).

cpp

#include <HX711.h>

#define DOUT_PIN 2

#define CLK_PIN  3

HX711 scale;

float calibration_factor = -7050; // Typical for 10kg cells. You WILL need to calibrate this.

void setup() {

Serial.begin(9600);

scale.begin(DOUT_PIN, CLK_PIN);

scale.set_scale(calibration_factor);

scale.tare(); // Zero the scale on startup

}

void loop() {

// Listen for 'T' command from Python app

if (Serial.available() > 0) {

char cmd = Serial.read();

if (cmd == 'T' || cmd == 't') {

scale.tare();

Serial.println("TARE_COMPLETE");

}

}

if (scale.is_ready()) {

float grams = scale.get_units();

Serial.print("WEIGHT: ");

Serial.println(grams, 1); // 1 decimal place

} else {

Serial.println("HX711_NOT_READY");

}

delay(100); // ~10 readings/sec

}

📌 Important: The calibration_factor will be wrong out of the box. You must calibrate it (see step 5).

### 🖥️ 3. PC & Python Setup

- Install pyserial (required by your app):

bash

1

- Connect Arduino via USB to your computer.

- Open Device Manager (Windows) or run ls /dev/tty* (macOS/Linux) to note the COM port (e.g., COM3, /dev/ttyUSB0, or /dev/cu.usbmodem...).

### 🔗 4. Connecting to the App

- Launch your Python timetable app.

- Click ⚖️ Schoolbag Weight Monitor.

- Click Refresh → select your Arduino’s COM port from the dropdown.

- Click Connect.

  - Status should change to Status: Connected to COMx

  - The live weight should appear in large numbers.

- Click Tare to zero the scale with your empty bag on it.

### ⚖️ 5. Calibration (Crucial Step)

The Arduino will not show accurate grams until calibrated.

- Place a known weight on the load cell (e.g., a 1000g water bottle, or a pack of rice with known weight).

- Open the Arduino IDE Serial Monitor (set baud to 9600).

- Note the WEIGHT: XXXX.X value it prints.

- Calculate new calibration factor:

1

- Update calibration_factor in the Arduino code, re-upload, and repeat until it matches closely.

### 🛠️ Troubleshooting Checklist

| Issue | Fix |
| --- | --- |
| App says Failed to open port | Another program (Arduino Serial Monitor, Cura, etc.) is using the port. Close it. |
| Weight shows 0.0 or jumps wildly | Check HX711 wiring. Ensure A+/A- aren’t swapped. Add a 100ms delay in loop() if missing. |
| App ignores readings | Arduino baud rate must be exactly 9600. Python expects WEIGHT: (with space). |
| Tare doesn’t work | Ensure you’re sending exactly T (uppercase). The code handles t too. |
| Weight in kg instead of g | The app expects grams. Calibrate your HX711 to output grams, not kilograms. |

Once calibrated and connected, the app will automatically:

- Read live weight via serial

- Compare it to the calculated expected bag weight

- Trigger the ±1000g warnings exactly as designed